For 72” panels with “Z” clip

The aluminum “Z” clip is purchased with the panels and should be packed with shipment.

“Z” clip for the wall is provided in 6’ lengths. May need to be trimmed to length for your project’s specifications, factory will provide the total Linear footage required.

# “Z” clip for the panels is provided in 4” sections. 4 for each panel, 2 for top, 2 for bottom. Screw for mounting these will be included.

To install pads at 4” above the finished floor:

Strike a chalk line along the wall at 6 1/2” from the floor (bottom edge of z-clip will go here.)

Then 72” from the floor for the top strip. Lower edge of z-clip will go on this line.

(See Drawing)

Attach 6’ lengths of “Z” clip securely to the wall with proper hardware. This hardware IS NOT provided by the factory.

On the Pads, from the top mark a line at 1-1/4”. (The top edge of the z-clip will go here)

Then from the bottom of the pad mark a line at 5-3/4”. (The top edge of the z-clip will go here)

Mount each piece of z-clip about 1” in from the edge of the pad.

Begin to hang the panels, corners & columns first then flat pads, making sure they are securely set into the clip at both the top and the bottom.

Using a wood block and a hammer tap firmly down on the top edge of the wall pad to help seat panel into the clip. Check plumb after every panel. Continue installing pads in a clockwise manner around the room until finished.

For additional installation support call the factory @: 800-338-6287